LIMO INSTRUCTION SET ARCHITECTURE (ISA) SPECIFICATION

1. Introduction

2. A1 — Processor Style

32-bit loadstore architecture, 32 registers, derived from RV321

log2(32) = 5 bit instruction

2.1 Architecture Overview

LIMO is a 32-bit load–store processor derived from RISC-V RV32I. As a load–store architecture, arithmetic and logical operations are performed on values held in registers, while memory is accessed only through explicit load and store instructions.

LIMO uses byte-addressed memory, where each memory address identifies one byte. A 32-bit word consists of four bytes and occupies four consecutive byte addresses. Word accesses are word-aligned, meaning that the starting address of a 32-bit word must be a multiple of four.

LIMO uses little-endian byte ordering, where the least significant byte of a multi-byte value is stored at the lowest memory address.

All LIMO instructions have a fixed width of 32 bits and use formats derived from RV32I. The selected LIMO instruction set uses the R-type, I-type, S-type and B-type instruction formats.

2.2 Design Rationale

LIMO retains the 32-bit load–store structure of RV32I to provide a simple and regular processor design. Restricting memory access to load and store instructions separates memory operations from arithmetic and logical operations and keeps instruction behaviour straightforward.

Fixed 32-bit instructions provide a consistent instruction length, simplifying instruction fetching and decoding. The use of RV32I-derived R, I, S and B formats also allows different instruction classes to share consistent field positions where possible.

Byte-addressed memory allows individual bytes to have unique addresses, while word alignment provides a regular organisation for 32-bit data. Little-endian byte ordering is retained from the RV32I architecture.

LIMO therefore keeps the required RV32I-derived processor characteristics rather than introducing unnecessary departures, while adapting the assembly language through Sesotho mnemonics.

3. A2 — Register Architecture

3.1 Register File Size

LIMO uses 32 general-purpose registers, numbered x0 to x31, with each register being 32 bits wide. Since there are 32 registers, a 5-bit register identifier is required:

log2(32)=5

Therefore, the rd, rs1 and rs2 fields in LIMO instructions are each 5 bits wide.

A 32-register file was selected because it provides greater flexibility and keeps LIMO close to the RV32I architecture from which it is derived. An 8-register design was rejected because it would provide too few registers for general computation and would create high register pressure. A 16-register design would reduce register-field width to 4 bits, but would provide fewer working registers. The 32-register design therefore provides a suitable balance between encoding cost and register availability

3.2 Register Definitions

| Registers | LIMO Names | Role |
| --- | --- | --- |
| x0 | lehe | Hard-wired zero |
| x1 | letsibolo | Return address |
| x2 | mokou | Stack pointer |
| x3 | sephara | Global pointer |
| x4 | khoele | Thread pointer |
| x5–x7 | nakoana1–nakoana3 | Temporary registers |
| x8–x9 | letsete0–letsete1 | Saved registers |
| x10–x17 | sephetho0–sephetho7 | Argument/result registers |
| x18–x27 | letsete2–letsete11 | Saved registers |
| x28–x31 | nakoana4–nakoana7 | Temporary registers |

3.3 Special-Purpose Registers

| Register | LIMO Name | Purpose |
| --- | --- | --- |
| PC | [Sesotho name to be decided] | Program counter; holds the address of the current/next instruction |
| x0 | lehe | Hard-wired zero; always contains the value 0 |
| x1 | letsibolo | Return address; stores the address used when returning from a procedure |
| x2 | mokou | Stack pointer; points to the current top of the stack |
| x3 | sephara | Global pointer |
| x4 | khoele | Thread pointer |

3.4 Design Rationale

LIMO uses 32 general-purpose registers to provide sufficient storage for operands, temporary values, procedure arguments, results and saved values while maintaining compatibility with the general register structure of RV32I. The use of 32 registers requires 5-bit register identifiers, which fit naturally within the selected 32-bit instruction formats.

Sesotho register names were selected to make the programmer-visible architecture consistent with the goal of creating a Sesotho-language ISA. Registers with similar purposes are grouped under common naming conventions, such as nakoana for temporary registers, letsete for saved registers and sephetho for argument/result registers. Special registers such as lehe, letsibolo and mokou are given distinct names because they perform specific architectural roles.

This organisation preserves the practical register roles of RV32I while presenting them through the LIMO naming system.

4. A3 — Instruction Set

4.1 Arithmetic Instructions

4.2 Logic Instructions

4.3 Memory Instructions

4.4 Branch and Jump Instructions

5. A4 — Sesotho Assembly Language

5.1 Instruction Glossary

5.2 Assembly Syntax

5.3 Apostrophe Handling

5.4 Case Sensitivity

6. A5 — Instruction Encoding

LIMO uses fixed 32-bit instructions derived from the RV32I instruction set architecture. The instruction set uses R-type, I-type, S-type and B-type formats and since LIMO has 32 general-purpose registers, 5 bits are required to identify each register because 25=32. To keep alignment with RV32I and simplify translation, implementation and testing, LIMO keeps the corresponding RV32I opcode and function-field encodings for the selected instructions

6.1 Instruction Formats

LIMO uses four instruction formats: R, I, S and B. All instructions are 32 bits wide.               Register fields are 5 bits wide, allowing each field to identify any of the 32 general-purpose registers

6.1.1 R-Type

R-type is used for register-to-register arithmetic and logical operations. It contains one destination register (rd) and two source registers (rs1 and rs2). The funct3 and funct7 fields distinguish the exact operation, while the opcode identifies the instruction family.

31          25 24      20 19      15 14    12 11       7 6          0

+--------------+----------+----------+---------+-----------+------------+

|    funct7    |   rs2    |   rs1    | funct3  |    rd     |   opcode   |

+--------------+----------+----------+---------+-----------+------------+

7 bits       5 bits     5 bits    3 bits     5 bits       7 bits

6.1.2 I-Type

I-type is used when an instruction requires an immediate value rather than a second source register. It is also used by load instructions and jalr. The immediate field occupies 12 bits.

31                      20 19      15 14    12 11       7 6          0

+--------------------------+----------+---------+-----------+------------+

|        imm[11:0]         |   rs1    | funct3  |    rd     |   opcode   |

+--------------------------+----------+---------+-----------+------------+

12 bits             5 bits    3 bits     5 bits       7 bits

For normal signed I-type immediate operations, the 12-bit immediate is sign-extended to 32 bits before use.

6.1.3 S-Type

S-type is used for store instructions. LIMO uses this format for boloka (SW). A store has no destination register because its result is written to memory rather than to a register.

31          25 24      20 19      15 14    12 11       7 6          0

+--------------+----------+----------+---------+-----------+------------+

|  imm[11:5]   |   rs2    |   rs1    | funct3  | imm[4:0]  |   opcode   |

+--------------+----------+----------+---------+-----------+------------+

7 bits       5 bits     5 bits    3 bits     5 bits       7 bits

rs1 contains the base register used to form the memory address, while rs2 contains the data to be stored. The immediate is divided into imm[11:5] and imm[4:0].Splitting the immediate allows rs1 and rs2 to remain in the same positions used by the other instruction formats, simplifying the hardware decoder.

6.1.4 B-Type

B-type is used for conditional branch instructions. A branch compares two registers and changes the next instruction executed when its condition is satisfied.

31                     25 24      20 19      15 14    12 11                 7 6       0

+------------------------+----------+----------+---------+--------------------+---------+

|    imm[12|10:5]        |   rs2    |   rs1    | funct3  |   imm[4:1|11]     | opcode  |

+------------------------+----------+----------+---------+--------------------+---------+

7 bits            5 bits     5 bits    3 bits          5 bits         7 bits

The B-type immediate represents a PC-relative offset, not an absolute address. Therefore, the branch target is formed relative to the current Program Counter (PC).The immediate is distributed across two fields. This arrangement keeps the sign bit at bit 31, allowing sign extension to begin early during instruction decoding.

6.2 Opcode and Function Assignments

| LIMO mnemonic | RV32I operation | Format | Opcode | funct3 | funct7 |
| --- | --- | --- | --- | --- | --- |
| kopanya | ADD | R | 0110011 | 000 | 0000000 |
| tlosa | SUB | R | 0110011 | 000 | 0100000 |
| kapa | OR | R | 0110011 | 110 | 0000000 |
| le | AND | R | 0110011 | 111 | 0000000 |
| eketsa | ADDI | I | 0010011 | 000 | — |
| suthumetsa | SLLI | I | 0010011 | 001 | special immediate/shift field |
| bofa | LW | I | 0000011 | 010 | — |
| qhoma | JALR | I | 1100111 | 000 | — |
| boloka | SW | S | 0100011 | 010 | — |
| lekana | BEQ | B | 1100011 | 000 | — |
| ekholo | BGE | B | 1100011 | 101 | — |

6.3 Immediate Ranges

-211≤immediate≤211-1

Meaning:  -2048 to 2048

| Instruction | Immediate use | Range |
| --- | --- | --- |
| eketsa (ADDI) | signed constant | −2048 to +2047 |
| bofa (LW) | base-address displacement | −2048 to +2047 bytes |
| boloka (SW) | base-address displacement | −2048 to +2047 bytes |
| qhoma (JALR) | address displacement | −2048 to +2047 bytes |
| suthumetsa (SLLI) | shift amount in RV32 | 0–31 |
| lekana (BEQ) | PC-relative branch displacement | −4096 to +4094 bytes, even offsets |
| ekholo (BGE) | PC-relative branch displacement | −4096 to +4094 bytes, even offsets |

6.4 Hand-Encoding Examples

Step 1: Identify the format

Step 2: Look up the fields

Step 3: Convert the registers

Step 4: Assemble the fields in order.

6.4.1 R-Type Example-kopanya

Instruction: kopanya x7, x5, x6

RV32I equivalent: add x7, x5, x6

Operation: x7 = x5 + x6

Step 1: Identify the format

kopanya is a register-to-register arithmetic instruction; therefore, it uses the R-type format.

Step 2: Identify the instruction fields

funct7 = 0000000

funct3 = 000

opcode = 0110011

Step 3: Convert the registers to 5-bit binary

rd  = x7 = 00111

rs1 = x5 = 00101

rs2 = x6 = 00110

Step 4: Assemble the fields

funct7   rs2    rs1    funct3   rd      opcode

0000000  00110  00101  000      00111   0110011

32-bit binary:

00000000011000101000001110110011

Hexadecimal:

0x006283B3

6.4.2 I-Type Example-eketsa

Instruction: eketsa x7, x5, 12
RV32I equivalent: addi x7, x5, 12
Operation: x7 = x5 + 12

Step 1: Identify the format

eketsa uses an immediate constant; therefore, it uses the I-type format.

Step 2: Identify the instruction fields

imm[11:0] = 000000001100

funct3    = 000

opcode    = 0010011

Step 3: Convert the registers to 5-bit binary

rs1 = x5 = 00101

rd  = x7 = 00111

Step 4: Assemble the fields

imm[11:0]    rs1    funct3   rd      opcode

000000001100 00101  000      00111   0010011

32-bit binary:

00000000110000101000001110010011

Hexadecimal:

0x00C28393

6.4.3 S-Type Example-boloka

Instruction: boloka x7, 8(x5)
RV32I equivalent: sw x7, 8(x5)
Operation: Store the 32-bit word in x7 at memory address x5 + 8.

Step 1: Identify the format

boloka is a store instruction; therefore, it uses the S-type format.

Step 2: Convert and split the immediate

The decimal immediate 8 is:

000000001000

The S-type format splits the immediate into two fields:

imm[11:5] = 0000000

imm[4:0]  = 01000

Step 3: Convert the registers to 5-bit binary

rs1 = x5 = 00101

rs2 = x7 = 00111

The remaining fields are:

funct3 = 010

opcode = 0100011

Step 4: Assemble the fields

imm[11:5] rs2    rs1    funct3  imm[4:0] opcode

0000000   00111  00101  010     01000    0100011

32-bit binary:

00000000011100101010010000100011

Hexadecimal:

0x0072A423

6.4.4 B-Type Example-lekana

Instruction: lekana x5, x6, 8
RV32I equivalent: beq x5, x6, 8
Operation: If x5 = x6, branch using a PC-relative displacement of 8 bytes.

Step 1: Identify the format

lekana is a conditional branch; therefore, it uses the B-type format.

Step 2: Encode the branch immediate

For a branch displacement of 8 bytes:

imm[12]   = 0

imm[10:5] = 000000

imm[4:1]  = 0100

imm[11]   = 0

Step 3: Convert the registers to 5-bit binary

rs1 = x5 = 00101

rs2 = x6 = 00110

The remaining fields are:

funct3 = 000

opcode = 1100011

Step 4: Assemble the fields

imm[12] imm[10:5] rs2    rs1    funct3 imm[4:1] imm[11] opcode

0       000000     00110  00101  000    0100     0       1100011

32-bit binary:

00000000011000101000010001100011

Hexadecimal:

0x00628463

6.5 Encoding Design Rationale

LIMO uses fixed 32-bit instructions to maintain a simple and regular instruction structure. The ISA contains 32 general-purpose registers, therefore each register identifier requires 5 bits because 25=32.

LIMO uses R-type for register-to-register arithmetic and logical operations because these instructions require two source registers and one destination register. I-type is used for instructions involving immediate values, loads and indirect jumps because it provides a 12-bit immediate field while retaining source and destination register fields.

S-type is used for store instructions. Since a store writes its result to memory rather than to a destination register, no rd field is required. Instead, the immediate is split into two fields while rs1 and rs2 remain in consistent positions. This maintains regularity in the instruction formats and simplifies instruction decoding.

B-type is used for conditional branches. Its immediate represents a PC-relative displacement rather than an absolute address. The immediate bits are distributed across the instruction while the sign bit remains at bit 31, supporting efficient sign extension during decoding.

LIMO retains the corresponding RV32I encoding structure for its selected instructions. This keeps LIMO closely related to the RISC-V architecture on which it is based and simplifies translation between LIMO and RV32I for implementation, testing and verification.

7. A6 — Design Decision Log

7.1 Register File Size

LIMO uses 32 general-purpose registers. This provides more registers for keeping temporary values, variables and procedure data, reducing register pressure and the need to move values frequently between registers and memory. The cost is that each register identifier requires 5 bits, since 25=32. Therefore, instructions containing rd, rs1 and rs2 require 5-bit register fields. The forwarding unit must also compare 5-bit register identifiers when detecting dependencies between instructions. A smaller register file, such as 16 registers, would require only 4-bit identifiers and narrower forwarding comparisons, but would increase register pressure. In our sample programs, 32 registers allow several working values to remain in registers without unnecessary memory accesses.

7.2 Architectural vs Microarchitectural Registers

7.3 Branch Resolution

7.4 Load-Use Hazard

7.5 Flags Register

7.6 Scaling Limitations

8. Sample LIMO Programs

8.1 Program 1

8.2 Program 2

8.3 Program 3